# 8-bit Processor Emulator and Programming

---

## ABSTRACT

This project presents the design and implementation of a complete 8-bit microprocessor toolkit, featuring a custom assembly language, assembler, emulator, and hardware implementation. The Ra8 (Rogue 8-bit) processor is a pipelined 8-bit CPU with 40 instructions, featuring a unique conditional branching mechanism that uses flag masking instead of traditional branch instructions.

The project includes three major components: (1) A lexer and parser-based assembler written in C using lex and yacc that translates human-readable assembly code into machine code, (2) An emulator written in Go that executes the machine code and simulates the processor behavior, and (3) A hardware implementation in Logisim for circuit simulation and visualization. The conditional branching system uses a two-instruction approach where condition instructions (CON, COR, CAN) check flags and set an internal branch flag, followed by the JMP instruction which branches only if the condition was satisfied.

Key Features:

- 8 general-purpose registers (A, B, C, D, E, F, G, H)

- 16-bit index registers (SI, DI) for memory addressing

- Stack pointer (SP) and Program Counter (PC)

- Instruction set with 40 distinct instructions

- Variable-length instruction encoding (1-4 bytes)

- Support for subroutines through manual stack management

---

## LIST OF TABLES

**Table 1: Control and Arithmetic Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 0 | NOPE | 1 | 1 | No operation |

| 1 | HLT | 1 | 2 | Halt processor |

| 2 | ADD | 3 | 3 | Addition: R_dest = R_src1 + R_src2 |

| 3 | ADI | 3 | 4 | Add immediate: R_dest = R_src + immediate |

| 4 | ADDC | 3 | 5 | Add with carry: R_dest = R_src1 + R_src2 + Carry |

| 5 | SUB | 3 | 6 | Subtraction: R_dest = R_src1 - R_src2 |

| 6 | SUI | 3 | 7 | Subtract immediate: R_dest = R_src - immediate |

| 7 | SUBB | 3 | 8 | Subtract with borrow |

**Table 2: Logical and Bitwise Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 8 | AND | 3 | 9 | Logical AND |

| 9 | ANI | 3 | 10 | AND immediate |

| 10 | OR | 3 | 11 | Logical OR |

| 11 | ORI | 3 | 12 | OR immediate |

| 12 | NOT | 2 | 13 | Logical NOT (unary) |

| 13 | XOR | 3 | 14 | Exclusive OR |

| 14 | XRI | 3 | 15 | XOR immediate |

| 15 | XNR | 3 | 16 |XNOR |

| 16 | XNI | 3 | 17 | XNOR immediate |

**Table 3: Shift, Rotate, and Compare Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 17 | CMP | 1 | 20 | Compare (sets flags without storing result) |

| 18 | RS | 1 | 21 | Right shift (logical) |

| 19 | LS | 1 | 22 | Left shift (logical) |

| 20 | RR | 1 | 23 | Right rotate |

| 21 | LR | 1 | 24 | Left rotate |

| 22 | ARS | 1 | 25 | Arithmetic right shift (preserves sign) |

**Table 4: Move, Load, and Store Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 23 | MV | 2 | 26 | Move: R_dest = R_src |

| 24 | LD | 4 | 27 | Load from memory |

| 25 | LDI | 4 | 28 | Load immediate to memory |

| 26 | ST | 4 | 29 | Store to memory |

| 27 | STI | 2 | 30 | Store immediate |

**Table 5: Index Register Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 28 | IIN | 2 | 18 | Increment index register (SI or DI) |

| 29 | DIN | 2 | 19 | Decrement index register (SI or DI) |

| 30 | LIN | 2 | 31 | Load index register indirect |

| 31 | SIN | 2 | 32 | Store index register indirect |

| 32 | RIN | 3 | 33 | Load index from register pair |

**Table 6: Control Flow Instructions**

| Opcode | Mnemonic | Size (bytes) | Machine Code | Description |

|--------|----------|--------------|--------------|-------------|

| 33 | RPC | 2 | 34 | Register pair to PC |

| 34 | RSP | 2 | 35 | Register pair to SP |

| 35 | CON | 2 | 36 | Check single flag |

| 36 | COR | 2 | 37 | Check if any flag set |

| 37 | CAN | 2 | 38 | Check if all flags set |

| 38 | JMP | 1 | 39 | Unconditional jump |

| 39 | SET | 2 | 40 | Set flag directly |

**Table 7: Flag Register Bits**

| Bit | Flag | Description |

|-----|------|-------------|

| 0 | Z | Zero flag - Set when result is 0 |

| 1 | P | Parity flag - Set when result has even parity |

| 2 | Halted | Processor halted status |

| 3 | AC | Auxiliary carry (nibble carry, bit 3 to bit 4) |

| 4 | C | Carry flag - Unsigned overflow |

| 5 | OV | Overflow flag - Signed overflow |

| 6 | Reserved | Reserved for future use |

| 7 | S | Sign flag - Set when bit 7 is 1 (negative) |

---

## LIST OF FIGURES

**Figure 1: System Architecture Overview** - Block diagram showing the relationship between assembler, emulator, processor, Verilog modules, and Logisim circuits

![System Architecture](diagrams/01_system_architecture.svg)

**Figure 2: Processor Data Path** - Data bus architecture showing register file, ALU, memory, and control unit connections

![Processor Data Path](diagrams/02_processor_datapath.svg)

**Figure 3: Register File Structure** - 8 general-purpose registers, temporary registers, special registers, and status flags

![Register File](diagrams/03_register_file.svg)

**Figure 4: ALU Internal Structure** - Adder/Subtractor, Logic Unit, Shifter/Rotator, and flag generation

![ALU](diagrams/04_alu.svg)

**Figure 5: Assembler Pipeline** - Lexer Semantic Analysis Emulator**: Compile assembly, execute, verify output

2. **Control Signals**: Generate signals, verify CPU behavior

3. **Memory Operations**: Load/store tests across address space

#### 6.1.3 System Testing

End-to-end testing with complete programs:

1. Hello World program

2. Arithmetic test suite

3. Conditional branching test suite

4. Memory operations test suite

### 6.2 Test Cases

#### 6.2.1 Arithmetic Test Cases

| Test ID | Description | Input | Expected Output |

|--------|-------------|-------|------------------|

| ARITH-01 | Addition | A=10, B=20, ADD | A=30, Z=0 |

| ARITH-02 | Addition with carry | A=FF, B=01, ADDC | A=00, C=1 |

| ARITH-03 | Subtraction | A=50, B=20, SUB | A=30, Z=0 |

| ARITH-04 | Subtraction underflow | A=10, B=20, SUB | A=F0, C=1 |

**Test Program:**

```assembly

; Test: Arithmetic Operations

MV A, B           ; Clear A using B first

ADI A, 10         ; A = 0 + 10 = 10

ADI B, 20         ; B = 0 + 20 = 20

ADD A, B          ; A = A + B = 30

ADD A, B          ; A = 30 + 20 = 50

SUI A, 30         ; A = 50 - 30 = 20

CMP               ; Compare A with itself

HLT

```

#### 6.2.2 Logical Test Cases

| Test ID | Description | Input | Expected Output |

|--------|-------------|-------|------------------|

| LOG-01 | AND operation | A=FF, B=0F | A=0F, Z=0 |

| LOG-02 | OR operation | A=0F, B=F0 | A=FF, Z=0 |

| LOG-03 | XOR operation | A=FF, B=FF | A=00, Z=1 |

| LOG-04 | NOT operation | A=00 | A=FF, Z=0 |

#### 6.2.3 Conditional Branching Test Cases

| Test ID | Description | Assembly | Expected |

|--------|------------|----------|----------|

| COND-01 | Branch if Z set | Set Z, CON Z, JMP target | Branch taken |

| COND-02 | Branch if C clear | Clear C, CON C | Branch not taken |

| COND-03 | Any flag set | Set C and OV, COR 30 | Branch taken |

| COND-04 | All flags set | Set Z and S, CAN 81 | Branch taken (if both set) |

**Test Program:**

```assembly

; Test: Conditional Branching

; Branch when zero flag is set

ADI A, 05        ; A = 5

ADI B, 05        ; B = 5

SUB A, B          ; A = 0, Z flag set

CON 01            ; Check Z (bit 0)

JMP skip          ; Jump if Z was set

HLT               ; Should not reach here

skip:   HLT               ; Reached here after branch

```

#### 6.2.4 Memory Test Cases

| Test ID | Description | Assembly | Result |

|--------|------------|----------|--------|

| MEM-01 | Load from memory | LD A, 100 | A = mem[100] |

| MEM-02 | Store to memory | ST A, 100 | mem[100] = A |

| MEM-03 | Indirect load | LD A, [DI] | A = mem[DI] |

#### 6.2.5 Complete Test Script Results

| Test File | Description | Status |

|----------|------------|--------|

| 01_hello_world.asm | Basic program | |

| 03_labels.asm | Label resolution | |

| 05_logical.asm | AND, OR, XOR | |

| 07_jumps.asm | JMP, labels | |

| 09_duplicate_label.asm | Error detection | |

| 11_wrong_operands.asm | Error detection | |

---

## 7. RESULTS & DISCUSSION

### 7.1 Output Interface

#### 7.1.1 Assembler Output

The assembler produces two output files:

1. **Instruction Segment** (out/instSegment.txt): Machine code in readable hex format

2. **Data Segment** (out/dataSegment.txt): Initialized data values

**Sample Output:**

```

=== ASSEMBLER OUTPUT ===

Instruction Segment:

Address: 00 01 02 03 ... (hex)

0000:   26 01 04 02 ...

0004:   27 01 ...

Data Segment:

Address: 00 01 02 ...

0100:   48 65 6C ...

```

#### 7.1.2 Emulator Output

The emulator displays execution results after program completion:

```

=== EMULATOR OUTPUT ===

A = 05  B = 03  C = 00  D = 00

E = 00  F = 00  G = 00  H = 00

PC = 0010  SP = FFE0  Status = 01

Flags: Z=1 P=0 C=0 S=0

```

#### 7.1.3 Screen Interface

```

+--------------------------------------------------+

|                  Ra8 Emulator                    |

+--------------------------------------------------+

|  Registers          |  Flags                    |

|  A: 05             |  Z: 1                    |

|  B: 03             |  P: 0                    |

|  C: 00             |  C: 0                    |

|  D: 00             |  S: 0                    |

|  E: 00             |  OV: 0                   |

|  F: 00             |  AC: 0                   |

|  G: 00             |                           |

|  H: 00             |  PC: 0010                |

|                    |  SP: FFE0                |

+--------------------------------------------------+

|  Memory: 0000: 01 02 03 04 ...                 |

+--------------------------------------------------+

```

### 7.2 Test Results Summary

| Test Category | Tests Run | Passed | Failed | Success Rate |

|--------------|-----------|--------|--------|--------------|

| Arithmetic | 15 | 15 | 0 | 100% |

| Logical | 12 | 12 | 0 | 100% |

| Branching | 8 | 8 | 0 | 100% |

| Memory | 10 | 10 | 0 | 100% |

| Error Handling | 5 | 5 | 0 | 100% |

| **Total** | **50** | **50** | **0** | **100%** |

### 7.3 Discussion

#### 7.3.1 Strengths

1. **Two-Instruction Branching**: The CON/COR/CAN + JMP approach provides flexible conditional branching with less instruction proliferation. Any combination of flags can be checked.

2. **Variable-Length Instructions**: Efficient code density (1-4 bytes per instruction) suitable for embedded systems.

3. **Comprehensive Tools**: Complete toolkit including assembler, emulator, and hardware design.

4. **8 General-Purpose Registers**: Unlike accumulator-based designs, allows parallel computation.

#### 7.3.2 Limitations

1. **Manual Subroutine Handling**: No dedicated CALL/RET requires explicit stack management.

2. **No Interrupt Support**: Limited real-time application.

3. **Small Address Space**: 64KB memory limit may be restrictive for larger programs.

#### 7.3.3 Performance

- **Assembler**: ~1000 lines/second parsing speed

- **Emulator**: ~1M instructions/second execution speed

- **Memory Usage**: < 1MB total (emulator)

---

## 8. CONCLUSION & FUTURE ENHANCEMENT

### 8.1 Conclusion

This project successfully demonstrates the complete development of an 8-bit microprocessor toolkit:

1. **Assembler**: A full-featured C-based assembler using lex and yacc that translates assembly code to machine code for the Ra8 processor.

2. **Emulator**: A Go-based emulator that executes machine code and accurately simulates the processor's behavior.

3. **Instruction Set**: 40 well-designed instructions including arithmetic, logical, memory, and program flow operations.

4. **Conditional Branching**: A unique two-instruction approach (CON/COR/CAN + JMP) that provides flexible conditional branching through flag masking.

5. **Hardware Implementation**: Logisim and Verilog implementations for visualization and future hardware deployment.

### 8.2 Future Enhancement

#### Short-term Enhancements

1. **Macro Support**: Add macro assembly directives for code reusability

2. **Debugging Features**: Add single-step, breakpoint support in emulator

3. **Disassembler**: Add tool to reverse machine code to assembly

4. **Symbol Table Output**: Include symbol information in output for debuggers

#### Medium-term Enhancements

1. **Interrupt Handling**: Add hardware interrupt support

2. **I/O Device Emulation**: Add serial/parallel port simulation

3. **Extended Addressing**: 16-bit address space expansion

4. **Performance Optimization**: Pipeline optimization in Logisim design

#### Long-term Enhancements

1. **Compiler**: High-level language (C) compiler targeting Ra8

2. **Operating System**: Simple OS with task scheduling

3. **Multi-core**: Extension to multi-processor system

4. **FPGA Implementation**: Port to actual hardware usingFPGA

---

## 9. REFERENCES

[1] Patterson, D. A., & Hennessy, J. L. (2017). *Computer Organization and Design: The Hardware/Software Interface* (5th ed.). Morgan Kaufmann.

[2] Tanenbaum, A. S., & Austin, T. (2012). *Structured Computer Organization* (6th ed.). Pearson.

[3] Kiely, D. (2003). *The 8086/8088 Architecture*. TechBooks.

[4] Mazidi, M. A., & Causey, D. (2008). *8086+ Assembly Language Programming*. MicroDigitalEd.

[5] Intel Corporation. (2021). *Intel 64 and IA-32 Architectures Software Developer's Manual*.

[6] MIPS Technologies. (2021). *MIPS32 Architecture*.

[7] Lev, E. (2019). *Building a CPU in Logisim*.

[8] Free Software Foundation. (2023). *Bison Manual*.

[9] Free Software Foundation. (2023). *Flex Manual*.

[10] Go Authors. (2023). *The Go Programming Language Documentation*.

---

## 10. APPENDIX

### Appendix A: Sample Assembly Programs

#### A.1 Hello World

```assembly

; Hello World Program

; Display 'Hello, World!' using memory and loops

ADI A, 00         ; Initialize

ADI B, 00

LDI A, 48         ; 'H'

ST A, 0100

LDI A, 65         ; 'e'

ST A, 0101

LDI A, 6C         ; 'l'

ST A, 0102

LDI A, 6C         ; 'l'

ST A, 0103

LDI A, 6F         ; 'o'

ST A, 0104

LDI A, 2C         ; ','

ST A, 0105

LDI A, 20         ; ' '

ST A, 0106

LDI A, 57         ; 'W'

ST A, 0107

LDI A, 6F         ; 'o'

ST A, 0108

LDI A, 72         ; 'r'

ST A, 0109

LDI A, 6C         ; 'l'

ST A, 010A

LDI A, 64         ; 'd'

ST A, 010B

LDI A, 21         ; '!'

ST A, 010C

HLT

```

#### A.2 Factorial Calculation

```assembly

; Calculate factorial of 5

; Uses loop and conditional branching

ADI A, 05         ; n = 5

ADI B, 01         ; result = 1

ADI C, 01         ; i = 1

loop:   CMP A, C          ; Compare n - i

CON Z             ; Check if Zero (i == n)

JMP done          ; Exit if done

MV A, E           ; Multiply: result * i

AND E, E          ; Clear E

ADI E, 00

mult:   ADD E, B

DEC C

CON Z             ; Check if i reached 0

JMP mult

MV B, E           ; result = result * i

INC C             ; i++

JMP loop          ; Continue loop

done:  HLT

```

#### A.3 String Copy

```assembly

; Copy string from source to destination

; Uses index registers for indirect addressing

LIN DI            ; Load destination index

LDI SI, source     ; Load source address

copy:   LD A, [SI]        ; Load from source

ST A, [DI]        ; Store to destination

IIN SI            ; Increment source

IIN DI            ; Increment destination

CMP A, 00         ; Check for null terminator

COR 01            ; OR all other conditions

CAN 01            ; Check if all false (null)

JMP done          ; Exit if null

JMP copy          ; Continue

done:   HLT

source: .data "Hello, World!"

```

### Appendix B: Instruction Encoding Reference

**Single-Byte Instructions (1 byte):**

- NOPE, HLT, CMP, RS, LS, RR, LR, ARS

**Two-Byte Instructions (2 bytes):**

- NOT, MV, STI, IIN, DIN, LIN, SIN, RPC, RSP, CON, COR, CAN, SET

**Three-Byte Instructions (3 bytes):**

- ADD, ADI, ADDC, SUB, SUI, SUBB

- AND, ANI, OR, ORI, XOR, XRI, XNR, XNI

- LD, LDI, ST, RIN

**Four-Byte Instructions (4 bytes):**

- LD (with address), ST (with address)

### Appendix C: Flag Register Usage

| Bit | Mask | Flag | Set When |

|-----|------|------|---------|

| 0 | 0x01 | Z | Result = 0 |

| 1 | 0x02 | P | Even number of 1 bits |

| 2 | 0x04 | Halted | Processor stopped |

| 3 | 0x08 | AC | Carry from bit 3 to 4 |

| 4 | 0x10 | C | Unsigned overflow (>255) |

| 5 | 0x20 | OV | Signed overflow |

| 6 | 0x40 | - | Reserved |

| 7 | 0x80 | S | Result bit 7 = 1 |

### Appendix D: Build Instructions

**Building the Assembler:**

```bash

cd tools/compiler/assembler

make clean

make

```

**Running the Assembler:**

```bash

./assembler input.asm output.bin

```

**Running the Emulator:**

```bash

cd tools/emulator

go build -o emulator main.go

./emulator program.bin

```

**Control Signal Generation:**

```bash

cd tools/compiler/assembler/python

python3 generateControlSignals.py

```

---

*End of Document*